Meghavi Kiosk System - Hardware & Integration Documentation

This document provides an in-depth guide to the hardware, software technologies, and configuration steps required to deploy the Meghavi Kiosk System in an airport environment for spa service ordering.

1. Technologies & Tools Used

Hardware

- 43" Portrait Touchscreen Monitor

- Windows-based Mini-PC

- Raspberry Pi Zero 2 W with Pi Camera v2

- HDMI and USB connections, USB power supplies

Operating Systems

- Windows 10 IoT or Windows 11 Pro (for the PC)

- Raspbian OS (for Raspberry Pi)

Programming Languages

- Python 3.9 or higher

Computer Vision

- OpenCV 4.x

- Ultralytics YOLOv8 for face detection (model from Hugging Face)

Web Framework

- Flask 2.x used on both the Windows PC and Raspberry Pi for serving content

UI Display Tools

- Python-webview 3.x for rendering full-screen screensaver interface

- Chromium or Microsoft Edge in kiosk mode for the ordering web app

GUI Library

- Tkinter is used for displaying the video download progress popup window

Packaging and Utilities

- zipfile, requests, tqdm for downloading and managing video content from the server

Miscellaneous Tools

- Task Scheduler (for automating app launch on Windows)

- Registry Editor and batch scripts (for customizing boot logo and auto-start behavior)

2. System Architecture

Fig : Architecture of Meghavi kioske system

3. Boot Logo Customization

By default, Windows displays its own logo during boot. To replace it with the Meghavi logo:

- Prepare Image: Create a logo.bmp at 720×400 px, 24-bit BMP.

- Use OEM Logo Tool (e.g., OEM Logo Changer):

  - Run tool as Administrator.

  - Select logo.bmp and apply to system partition.

- UEFI Method (for modern hardware):

  - Copy logo.bmp to C:\Windows\System32\oobe\info\backgrounds.

  - Edit Registry:

[HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\Windows\CurrentVersion\Authentication\LogonUI]

"OEMBackground"=dword:00000001

- Reboot and verify the Meghavi logo appears.

Note: Steps may vary by manufacturer. Consult hardware vendor documentation if OEM tool unsupported.

4. Auto-Start Ordering Application

To ensure the web application launches immediately after login:

A. Startup Folder Method

- Create a batch script start_meghavi.bat:

@echo off

timeout /t 5

start "" "C:\Program Files\Microsoft\Edge\Application\msedge.exe" --kiosk http://localhost:5000 --edge-kiosk-type=fullscreen.

- Place start_meghavi.bat in %APPDATA%\Microsoft\Windows\Start Menu\Programs\Startup.

- On next login, the kiosk brow ser opens automatically.

B. Task Scheduler Method

- Open Task Scheduler → Create Task.

- General: Run whether user is logged on or not; highest privileges.

- Triggers: At log on of any user.

- Actions: Start a program → Program/script: C:\Windows\System32\cmd.exe; Add arguments: /c start_meghavi.bat.

- Conditions: Uncheck “Start the task only if the computer is on AC power.”

- OK and enter credentials.

5. Distance Calibration Procedure

The system uses bounding‐box pixel area to estimate user distance using a power-law model:

Calibration Formula

The formula used for estimating the distance based on bounding box area is:

distance = a * (area ** b)

Where:

- distance is the estimated distance from the camera in centimeters (cm)

- area is the area of the detected face's bounding box in pixels (width × height)

- a and b are calibration coefficients obtained from curve fitting

A. Data Collection (caliberation.py)

- Ensure models/model.pt exists (downloaded YOLOv8 face model).

- Run:

- python caliberation.py --model_path models/model.pt --output calibration.json

- Steps in GUI:

  - Place face at a known distance (e.g., 50 cm) from camera.

  - Press c, input distance (cm).

  - Repeat for 10–15 distances (20 cm to 150 cm).

  - Press q to finish.

- Output: calibration.json includes list of {area, distance} pairs and computed constant K.

B. Curve Fitting (caliberation_fit.py)

- Run:

- python caliberation_fit.py

- Outputs:

- a = <value>

- b = <value>

- RMSE = <error> cm

- Copy a and b into meghavi_script.py:

- distance = a * (area_px ** b)

Fig: example calibration points done at different distances

6. Human Detection & Screensaver Logic

- Camera Stream: Pi Zero Flask app → http://<pi_ip>:5000/video.

- Main Script: meghavi_script.py on Windows:

  - Loads YOLO model (ultralytics).

  - Computes area_px → distance_cm.

  - If no face within 1 m for 10 s, launch screensaver:

open_screensaver()

  - If face reappears, call:

close_screensaver()

- Screensaver App:

  - screensaver.py: Uses webview.create_window(..., fullscreen=True).

  - Videos served via local app.py at http://localhost:5000.

7. Daily Video Update & Management

File: meghavi_functions.py

- Reads last update date from textFiles/date_txt.txt.

- Hits IDS_API_URL to fetch video IDs.

- Compares with previous IDs (textFiles/ids.txt).

- On change, downloads ZIP from ZIP_URL using download_zip_popup.py UI.

- Extracts videos, shuffles, and renames to videos/###_filename.

8. Flask Backend Setup (Windows)

File: app.py

@app.route('/')

def index():

return render_template('index.html')

@app.route('/videos/list')

def list_videos():

files = [f for f in os.listdir(VIDEO_FOLDER) if f.endswith(SUPPORTED_EXTENSIONS)]

return jsonify(files)

@app.route('/videos/<path:filename>')

def serve_video(filename):

return send_from_directory(VIDEO_FOLDER, filename)

- Ensure templates/index.html contains ordering UI.

- Place video files in videos/.

- Start server: python app.py

9. Maintenance & Troubleshooting

- Camera Change: Re-run Sections 5A–5B for new calibration.

- Boot Logo Fails: Verify image format & paths; consult OEM tool logs.

- App Auto-Start: Test batch/script manually; check Task Scheduler history.

- Network Issues: Ensure Pi and PC are on same LAN; open port 5000.